Measurement software calibration and use

- 1. Prepare microscope, calipers, measurement software

- Calipers can not be tilted (you can choose your own reference)

- 2. Calibration

- 1）Measure the distance of 1mm caliper on the chart as follows

- Bottom right corner Select Calibration Management --- Name it. For example, name it as 1.

3）Double click [CALIBRATION

As shown in the figure, the previous measurement value, has been calibrated to the actual distance of 1mm.

- 3.After the calibration is established, you can start the actual measurement

- Note: After the calibration is established, the measurement process, you can not adjust the focus (that is, you can not rotate the focus ring, you can lift the height.)

- Reason: The data measured at the same magnification is valid, if you adjust the focus ring, the magnification will change, different magnification, the measurement value will be inaccurate]

- After calibration, you can remove the calipers, the items to be observed into the measurement area for measurement

- As shown above, rectangle measurement (length, width, perimeter, area)